# Milma Vision-Based Packet Counting System

Site Survey Report

Site survey findings only. Design and implementation decisions are not covered.

# 1. Requirement as stated by client

- Count the number of milk packets inside each crate, on the crate conveyor.

- Purpose: obtain total packet production figures.

- Both crate conveyors are to be covered.

- Counts to be segregated by packet variant (colour).

- No numeric accuracy target given. Client stated only “maximum accuracy”.

# 2. Line data

| Parameter | Value |
| --- | --- |
| Crate conveyors | 1, running simultaneously |
| Conveyor type | Chain crate conveyor, with empty-crate return lines |
| Crates per minute, per line | 24 |
| Packets per crate | 20 |
| Packet variants | 3 — Orange, Violet, Green |
| Variant mixing within a crate | None. Each crate holds one variant only |
| Packet rate per line | 280 packets/min |
| Packet rate, both lines | NA |
| Stated daily throughput | 1.5 lakh packets/day, plant total |
| Shift 1 | 07:30 – 15:30 |
| Shift 2 | NA |
| Shift 3 | NA |
| Typical downtime cause | Wear and tear |

# 3. Mechanical measurements

| Parameter | Value |
| --- | --- |
| Conveyor belt height, floor to belt | 900 mm |
| Belt width | 450 mm |
| Usable crate width | 350 mm |
| Crate retention | Crate overhangs belt, retained by an outer guide frame |
| Max available mounting height, floor to structure | 950 mm (measured on open line) |
| Side guiderails on belt | Nil (outer frame only) |
| Vibration during operation | Minimal |
| Thermal obstructions | Nil |

# 4. Crate

| Parameter | Value |
| --- | --- |
| Type | Blue moulded plastic, Milma E.R.C.M.P.U. Ltd. branded |
| Length | 565 mm |
| Width | 300 mm |
| Height | 180 mm (internal vs external not distinguished) |
| Internal dividers | NIL |
| Crate spacing on conveyor | Crates travel abutting, no gap between consecutive crates |

# 5. Packet

Milma 500 ml pillow pouch.

| Parameter | Value | Source |
| --- | --- | --- |
| Film web width | 325 mm | LDPE film specification |
| Film gauge | 53 µm | LDPE film specification |
| Lay-flat width | 150 mm | Estimate, not measured |
| Filled length | 150 mm | Estimate, not measured |
| Filled thickness | ~35–45 mm | Estimate, not measured |

Film web width and gauge are reel specifications, not packet dimensions. No packet was physically measured on site.

Packing arrangement: packets lie flat and shingled — each pouch overlapping the next in a regular run, 14 per lane, 2 lanes.

Fin-seal corners stand proud of the pouch body. Packets touch along the run; no gap between packets.

# 6. Site conditions observed

- Lighting: overhead fluorescent tube fixtures only. Dim by machine-vision standards.

- Surroundings: walls and machine guards are polished stainless steel cladding, producing strong uneven specular glare.

- Packet appearance: glossy white translucent film with printed branding. In all photographs the film appears white with orange print; whether the four variants differ by film tint or print colour only was not established.

- Environment: wet washdown dairy area.

- Crate handling at line end: crates are manually lifted off the belt. Some crates receive a brief incidental stop before offload, others do not. Dwell time is variable and operator-dependent.

- Wall hatch: crates on each line pass through a wall aperture between rooms.

- Existing camera: a CCTV camera is wall-mounted near the hatch on one line. Not suitable for counting; noted as evidence that cable routing exists.

- Power and network: none currently available at the line. Client has confirmed these will be provided as required.